# Appendix — July 2026 South Asia flood: extent, depth and impact summary

Event window: 2026-07-23 to 2026-07-28.

Countries: Pakistan (PAK), Nepal (NPL), Bhutan (BTN), Bangladesh (BGD), Sri Lanka (LKA), India (IND).

# A. Methods

Flood extent was derived from a Google Earth Engine (GEE) multi-sensor ensemble of Sentinel-1 backscatter, Sentinel-2 NDWI and JRC surface-water products (~100 m; UNION product = any sensor detects flood). Flood depth was estimated with the boundary-elevation FwDET method (Cohen et al. 2018) on the ETOPO1 1 arc-minute DEM (~1.85 km). Population and Under-18 counts came from WorldPop 2025 (100 m); urban vs rural split from GHS-SMOD 2025 R2023A (Level 2 codes 21/22/23/30 = urban, 11/12/13 = rural). Road length used OSM 2025 country extracts, clipped by pixel-touch against a ~90 m rasterised flood mask (equal-area EPSG:6933 for length in km). Schools and hospitals were geolocated point layers sampled against the per-country FwDET depth raster. Deep-water hazard used JRC CEMS-GLOFAS RP100y (100-year return period) at thresholds ≥ 1.5 m and ≥ 2.0 m, intersected with the observed UNION extent on the JRC grid.

# B. Flood extent and depth per country

Total observed flooded area = 11,905 km² across the six countries.

| Country | Extent (km²) | Median depth (m) | Mean depth (m) | p95 depth (m) | Max depth (m) |
| --- | --- | --- | --- | --- | --- |
| Pakistan | 1,421 | 1.0 | 3.91 | 17.0 | 30.0 |
| Nepal | 83 | 4.0 | 8.12 | 25.0 | 30.0 |
| Bhutan | 8 | 19.0 | 16.88 | 28.8 | 30.0 |
| Bangladesh | 3,433 | 1.0 | 2.08 | 5.0 | 30.0 |
| Sri Lanka | 141 | 6.0 | 7.20 | 20.0 | 30.0 |
| India | 6,820 | 3.0 | 5.54 | 19.0 | 30.0 |
| TOTAL | 11,905 | — | — | — | — |

# C. Affected total population and Under-18

| Country | Total pop 2025 | Affected pop | % of country | U18 2025 | U18 affected | % of U18 |
| --- | --- | --- | --- | --- | --- | --- |
| Pakistan | 253,202,736 | 221,571 | 0.09% | 108,965,482 | 101,141 | 0.09% |
| Nepal | 29,628,159 | 14,699 | 0.05% | 9,940,201 | 5,512 | 0.06% |
| Bhutan | 793,943 | 262 | 0.03% | 203,253 | 76 | 0.04% |
| Bangladesh | 174,617,260 | 2,724,949 | 1.56% | 58,806,036 | 941,062 | 1.60% |
| Sri Lanka | 23,162,794 | 35,856 | 0.15% | 6,150,287 | 10,686 | 0.17% |
| India | 1,457,435,336 | 2,195,958 | 0.15% | 431,330,489 | 639,245 | 0.15% |
| TOTAL | 1,938,840,228 | 5,193,295 | 0.27% | 615,395,748 | 1,697,722 | 0.28% |

# D. Affected infrastructure — schools, hospitals, roads

Schools and hospitals counts are for facilities that fall inside an ETOPO1-grid cell classified as flooded by FwDET-July-2026. Road totals are OSM road-km whose vertices touch a ~90 m rasterised flood cell. "% of country" is the affected share of the pyogrio-estimated national road total.

| Country | Schools (FwDET) | Hospitals (FwDET) | Roads affected (km) | % country roads |
| --- | --- | --- | --- | --- |
| Pakistan | 14 | 21 | 10,745 | 0.92% |
| Nepal | 144 | 49 | 3,059 | 0.85% |
| Bhutan | 0 | 0 | 359 | 1.28% |
| Bangladesh | 154 | 94 | 26,285 | 5.28% |
| Sri Lanka | 109 | 33 | 2,345 | 0.94% |
| India | 197 | 1,703 | 49,347 | 1.34% |
| TOTAL | 618 | 1,900 | 92,140 | — |

# E. Urban vs rural share of affected population (GHS-SMOD)

| Country | Pop urban | Pop rural | Urban % | Rural % | U18 urban | U18 rural |
| --- | --- | --- | --- | --- | --- | --- |
| Pakistan | 103,315 | 117,584 | 46.63% | 53.07% | 44,676 | 56,186 |
| Nepal | 9,020 | 5,679 | 61.36% | 38.64% | 3,366 | 2,146 |
| Bhutan | 88 | 174 | 33.46% | 66.54% | 25 | 51 |
| Bangladesh | 2,368,393 | 347,338 | 86.92% | 12.75% | 813,842 | 123,691 |
| Sri Lanka | 28,971 | 6,511 | 80.80% | 18.16% | 8,688 | 1,884 |
| India | 1,567,388 | 624,547 | 71.38% | 28.44% | 462,444 | 175,734 |

# F. Deep-water hazard — JRC RP100y ∩ observed event

Population living where the 1-in-100-year JRC hazard depth is ≥ the threshold AND the observed UNION extent touches the same JRC grid cell. Because JRC cells (~1.85 km) are ~340× larger than observed cells (~100 m), the intersection area is an upper bound on 'deep-water exposure inside the event footprint'.

| Country | ≥ 1.5 m km² | ≥ 1.5 m pop | ≥ 1.5 m U18 | ≥ 2.0 m km² | ≥ 2.0 m pop | ≥ 2.0 m U18 |
| --- | --- | --- | --- | --- | --- | --- |
| Pakistan | 25,441 | 9,895,159 | 4,398,462 | 19,658 | 7,114,524 | 3,154,174 |
| Nepal | 2,757 | 573,969 | 192,322 | 2,534 | 523,145 | 172,398 |
| Bhutan | 571 | 24,661 | 6,703 | 568 | 24,661 | 6,703 |
| Bangladesh | 33,897 | 45,967,030 | 15,052,085 | 27,361 | 37,599,385 | 12,254,616 |
| Sri Lanka | 1,105 | 620,015 | 185,960 | 749 | 408,698 | 119,222 |
| India | 87,757 | 55,841,733 | 16,822,035 | 70,998 | 43,103,543 | 12,899,057 |

# G. PDMA Pakistan reporting-point validation (brief)

Eight lat/lon locations along the Ravi River corridor (Lahore / Narowal district, Punjab) reported by the Provincial Disaster Management Authority (PDMA), Punjab, were independently checked against three satellite layers: the GEE UNION extent (100 m), the FwDET depth (ETOPO1 grid), and the JRC RP100y hazard, plus a live Sentinel-2 NDWI change analysis over a ~1 km buffer per point comparing 2026-06-01 – 2026-07-15 (before) with 2026-07-16 – 2026-08-05 (after).

Result. All 8 PDMA points showed a strongly positive mean Sentinel-2 NDWI change (+0.10 to +0.37), confirming new or expanded surface water in the after window, consistent with the PDMA field reports. Two points (4 and 6) also fell on an ETOPO1 cell that the FwDET model classified as flooded (1.0 m depth); the other six locations were within the JRC RP100y flood-hazard footprint (0.23 – 1.60 m modelled depth). The GEE UNION multi-sensor extent product did not classify any of the eight PDMA points as flooded — indicating that short-duration rainfall pooling of the type reported here is under-detected by our current UNION thresholds; a shorter-interval NDWI-change trigger (Sentinel-2 pairs) is recommended for such sub-kilometre events. Full per-point evidence and before/after imagery are in NDMA_Pakistan/NDMA_Pakistan_July_2026_report.docx.